Rill Tech Race Prep & Parts
1336 Sherman Drive Unit A
Longmont, CO 80501
Tel: 303-776-8855
Cell: 303-350-6066
Email: Speedyrill@aol.com
Web site: www.Rilltechracing.com

Setting oil levels for Suzuki  Hayabusa  and GSXR 1000/750/600cc

Step 1:

- Fill oil to just over F mark on sight glass.

- Remove spark plugs, loosen oil filter so there is a small air gap between filter and case.

- Turn over engine until 1/3 of a quart of oil comes past oil filter. Then tighten oil filter.

Step 2:

- Top up oil to F mark on sight glass.

- Loosen top oil cooler hose and turn over engine until oil is seen at connection

- Retighten cooler hose. Turn engine again till you see oil pressure. Refit spark plugs.

Step 3:

- Top up oil again to top of F mark on sight glass.

- Start engine and run until oil temperature is at a minimum of 150F.

- Hold throttle at 3000 RPM for at least 10 seconds (oil level will noticeably drop)

- Look at sight glass and keep topping oil up until there is a small air bubble at the top of sight glass.

- Stop engine and pour in another 1/3 of a quart of oil , then you are ready to go racing.

ACCUSUMP USERS NOTE:

If your car is fitted with an Accusump, ensure that the Accusump is FULL when setting oil level.  If you have a MANUAL Accusump valve, this means turning the Accusump ON.  If you have an electric Accusump valve, turn the Accusump on initially, while oil pressure is high, but turn it off before oil pressure drops below your valve’s opening pressure (usually 35psi).

Setting oil level with an Accusump empty means that you will be LOW on oil by the size of your Accusump, which will almost surely cause oil pressure problems.